# 📘Digital Logic & Number Systems Question Bank

## 1. Number Systems

- Convert (1101)2,  (1101)_2 to decimal.

- Convert (347)8, (347)8 to decimal.

- Convert (2A5)16, (2A5)16 to binary.

- Convert (101101.101)2, (101101.101)2 to octal.

- Convert (375)10, (375)10 to hexadecimal.

- Perform (1011)2+ (1101)2, (1011)_2 + (1101)_2 using binary addition.

- Subtract (11001)2−(10110)2, (11001)_2 - (10110)_2 using the 2’s complement method.

## 2. Base Arithmetic

- Multiply (101)2 × (11)2,  (101)_2 x (11)_2 and express the result in binary and decimal.

- Add (235)8+(457)8, (235)_8 + (457)_8 and express the result in octal.

- Subtract (A6)16−(3B)16, (A6)_{16} - (3B)_{16} and express the result in hexadecimal.

- Multiply (24)10×(15)10(24)_{10} \times (15)_{10} and convert the result to binary.

- Perform (1A)16 + (1110)2, (1A)_{16} + (1110)_2. Express the answer in hexadecimal.

## 3. Logic Gates

- Draw the truth tables for AND, OR, NOT gates.

- Construct XOR gate using only NAND gates. Show truth table.

- Prove that NAND is a universal gate by constructing AND and OR using NAND.

- Design a circuit using logic gates that gives output 1 only when exactly two inputs out of A, B, C are 1.

- Write the truth table for (A+B)(A′+C)(A + B)(A′ + C).

- Simplify the Boolean expression: (A+B)(A′+C)(B+C)(A + B)(A′ + C)(B + C).

- Design the truth table for a Half-Adder and provide Boolean expressions for Sum and Carry.

## 4. Truth Tables & Expressions

- Write the truth table for the expression: F=A⋅B+A′⋅CF = A \cdot B + A′ \cdot C.

- Simplify using Boolean algebra: X+X′YX + X′Y.

- Reduce the expression: F=(A+B)(A+C)(B+C)F = (A + B)(A + C)(B + C).

- Construct the truth table for F=(A+B′)(B+C)F = (A + B′)(B + C).

- Simplify F(A,B,C)=Σ(0,1,2,5,6,7)F(A,B,C) = Σ(0,1,2,5,6,7) using Karnaugh Map.

- Prove De Morgan’s Laws using truth tables.

- Write the SOP (Sum of Products) form for the truth table where F=1F = 1 for minterms 1, 3, 5, 7.